MICROPROCESSOR AND MICROCONTROLLER-BASED DESIGNS

PROJECT FINAL REPORT

TEMPERATURE SENSOR USING MICROCONTROLLER

PROGRESS WEEK 1: DIGITAL THERMOMETER

EQUIPMENT USED:

- PIC18F452

- LM35 (Temperature sensor)

- Potentiometer 10k

- Resister 1k

- Wires

- LCD 16X2

- Crystal oscillator 4Mz

- Capacitors 22pf

COMPILER USED:

- The compiler we used in this project is MIKROC PRO 7.6. The reason is that it is the best compiler for microcontrollers in C language.

C LANGUAGE CODE:

// connections between lcd and pic sbit LCD_RS at RC3_bit; sbit LCD_EN at RC2_bit; sbit LCD_D4 at RC4_bit; sbit LCD_D5 at RC5_bit; sbit LCD_D6 at RC6_bit; sbit LCD_D7 at RC7_bit; //directions sbit LCD_RS_Direction at TRISC3_bit; sbit LCD_EN_Direction at TRISC2_bit; sbit LCD_D4_Direction at TRISC4_bit; sbit LCD_D5_Direction at TRISC5_bit; sbit LCD_D6_Direction at TRISC6_bit; sbit LCD_D7_Direction at TRISC7_bit; //prototype void mydelay(int); unsigned int AD_Read(); void ADC_ini(); char tp1[7]; char tp2[7]; unsigned int temp_celc; unsigned int temp_kel; void main() { TRISA=0xFF; lcd_Init(); mydelay(10); lcd_out(1,1,"Temperature is:-"); mydelay(10); delay_ms(500); while(1) { ADC_ini(); temp_celc= AD_read()*0.489; temp_kel=temp_celc + 273; IntTostr(temp_celc,tp1); IntTostr(temp_kel,tp2); Lcd_Cmd(_LCD_CLEAR); Lcd_cmd(_LCD_FIRST_ROW); Lcd_cmd(_LCD_MOVE_CURSOR_LEFT); mydelay(2); lcd_out(1,1,tp1); lcd_out(1,7," C"); mydelay(10); lcd_out(2,1,tp2); lcd_out(2,7,"K"); lcd_cmd(_LCD_CURSOR_OFF); mydelay(5000); } } void mydelay(int del) { int n=0; int b=0; for(; n<del; n++) { for(; b<1000; b++){int s=0;} } } void ADC_ini() { ADCON0=0b11010101; //RC OSC, CHANNEL 2, GO=1, ADC ON ADCON1=0b10000000; //Right justified, vref=5 volt mydelay(600); } unsigned int AD_read() { unsigned int ADC_RES=0; while(ADCON0.DONE=1){}; ADC_RES=ADRESL; ADC_RES¦=((unsigned int)ADRESH)<<8; return ADC_RES; }

PROGRESS WEEK 2: DIGITAL THERMOMETER

PROGRESS WEEK 3: DIGITAL THERMOMETER

PROGRESS WEEK 4: TEMPERATURE SENSOR

EQUIPMENT USED:

- Arduino-UNO

- LM35 (Temperature sensor)

- Resisters 100ohm

- Wires

- Red and Green Led

COMPILER USED:

- ARDUINO is a very handy and very powerful Integrated Development Editor that will let you write code as well as upload it to an Arduino board for testing purposes. This cross-platform IDE works along with an Arduino controller for writing, compiling as well as uploading the code to the board.

CODE:

#define LM35 A0 int GREEN=7; int RED=6; void setup() { Serial.begin(9600); pinMode (7,OUTPUT); pinMode (6,OUTPUT); } void loop() { float Imvalue = analogRead(LM35); float temp=(Imvalue*500)/1023; Serial.println(temp); //Temperature in clecius delay(1000); if (temp>60) { digitalWrite(GREEN,LOW); digitalWrite( RED,HIGH); } else { digitalWrite(GREEN,HIGH); digitalWrite(RED,LOW); delay (200); } }

CONNECTIONS:

1) Connect the Components as per the Schematic Circuit Diagram

2) Upload "Lm35.ino"

OUTPUT:

- Verify code

- Upload code

- Open screen monitor

SCREEN MONITOR OUTPUT:

PROTEUS SIMULATION:

Once we increase the temperature beyond the limit(30) Then led starts to glow.

Likewise, if we decrease temperature Led stops glowing.

LIMITATIONS:

The project does not give negative temperature just a low and high indication.

No Lcd display implemented.